Synchronous Boost Converter

Feedback Control Design

For Solar MPPT Applications

Based on EEL424C Power Electronics I Methodology

Design Specifications:

500W Output Power  |  25-60V Input  |  120V Output

Target Efficiency: 98-99%

Ruben Gonzalez-Vera

University of Florida

EEL424C Power Electronics I

# Table of Contents

1. Introduction and Design Specifications

2. Step 1: Steady-State Parameters

3. Step 2: Small-Signal Transfer Functions

4. Step 3: Bode Plot Analysis

5. Step 4: Compensator Design

6. Step 5: Loop Gain Verification

7. Step 6: Op-Amp Implementation

8. Step 7: Complete Circuit Schematic

9. Step 8: Simulation Verification

10. Key Differences: Buck vs. Boost Control Design

# 1. Introduction and Design Specifications

This document presents a complete feedback control design for a 500W synchronous boost converter intended for solar Maximum Power Point Tracking (MPPT) applications. The design methodology follows the 8-step process from EEL424C Power Electronics I, adapted from the buck converter project to address the unique challenges of boost converter control.

## 1.1 Design Specifications

| Parameter | Value |
| --- | --- |
| Output Power | 500 W |
| Input Voltage Range | 25V - 60V (Solar Panel MPP) |
| Nominal Input Voltage | 48V |
| Output Voltage | 120V DC Bus |
| Output Voltage Tolerance | ±1% |
| Switching Frequency | 150 kHz |
| Inductance (L) | 220 µH |
| Inductor DCR (R_L) | 15 mΩ |
| Output Capacitance (C) | 100 µF (Low ESR Film) |
| Load Resistance (R) | 28.8Ω (at full load) |
| PWM Modulator Amplitude (V_M) | 3.3V |
| Reference Voltage (V_ref) | 2.5V |
| Target Efficiency | 98-99% |

## 1.2 Control Design Requirements

- Loop-gain crossover frequency (f_c): Maximum achievable while maintaining stability

- Phase margin (φ_m): ≥ 45° (targeting 60° for robustness)

- Gain margin: ≥ 10 dB

## 1.3 Key Differences from Buck Converter Control

The boost converter presents several unique control challenges compared to the buck converter:

- Right-Half-Plane (RHP) Zero: The boost Gvd transfer function contains a RHP zero that adds phase lag, fundamentally limiting achievable bandwidth

- Non-minimum phase behavior: The RHP zero causes the output to initially move opposite to the final direction during transients

- Variable DC gain: The control-to-output gain varies with D² rather than linearly

- Higher Q factor typically: The boost topology often exhibits more pronounced resonance

# 2. Step 1: Steady-State Parameters

## 2.1 Boost Converter Topology

The synchronous boost converter replaces the diode with a synchronous MOSFET (low-side switch) to reduce conduction losses. During the ON interval (0 to DTs), the low-side switch conducts, storing energy in the inductor. During the OFF interval (DTs to Ts), the high-side switch conducts, transferring energy to the output.

## 2.2 Voltage Conversion Ratio

For an ideal boost converter, the voltage conversion ratio is:

M(D) = V / V_g = 1 / (1 - D) = 1 / D'

where D' = 1 - D is the complement of the duty cycle.

## 2.3 Duty Cycle Calculation

For the nominal operating point (V_g = 48V, V = 120V):

D = 1 - V_g/V = 1 - 48/120 = 0.6

D' = 1 - D = 0.4

## 2.4 Operating Range

| Condition | V_g (V) | D |
| --- | --- | --- |
| Minimum Input | 25 | 0.792 |
| Nominal Input | 48 | 0.600 |
| Maximum Input | 60 | 0.500 |

## 2.5 Inductor Current

The DC inductor current equals the input current:

I_L = I_g = P_out / (V_g × η) ≈ 500 / (48 × 0.98) = 10.63 A

## 2.6 CCM Verification

The inductor current ripple is:

ΔI_L = (V_g × D) / (L × f_s) = (48 × 0.6) / (220µH × 150kHz) = 0.873 A

Since I_L >> ΔI_L/2, the converter operates well within CCM.

## 2.7 Output Voltage Ripple

ΔV = (I_out × D) / (C × f_s) = (4.17 × 0.6) / (100µF × 150kHz) = 167 mV

This is 0.14% of the output voltage, well within the ±1% tolerance.

# 3. Step 2: Small-Signal Transfer Functions

## 3.1 Boost Converter Small-Signal Model

Using state-space averaging and linearization, the boost converter small-signal model yields three key transfer functions. The critical difference from the buck converter is the presence of a Right-Half-Plane (RHP) zero in Gvd.

## 3.2 Control-to-Output Transfer Function (G_vd)

The boost converter control-to-output transfer function is:

G_vd(s) = G_d0 × (1 - s/ω_z) / (1 + s/(Qω_0) + s²/ω_0²)

Where the parameters are:

- DC Gain: G_d0 = V / D' = 120 / 0.4 = 300 V

- Resonant frequency: ω_0 = D' / √(LC) = 0.4 / √(220µH × 100µF) = 8,528 rad/s

- f_0 = ω_0 / 2π = 1,358 Hz

- Quality factor: Q = D'R√(C/L) = 0.4 × 28.8 × √(100µF/220µH) = 7.77

- RHP Zero: ω_z = D'²R / L = (0.4)² × 28.8 / 220µH = 20,945 rad/s

- f_z = ω_z / 2π = 3,333 Hz

## 3.3 The Right-Half-Plane Zero - Critical Design Constraint

The RHP zero is the defining characteristic of boost converter control. Unlike a left-half-plane zero that adds phase lead, the RHP zero:

- Adds +20 dB/decade magnitude (same as LHP zero)

- Adds -90° phase (opposite of LHP zero!)

This means the RHP zero increases gain while simultaneously reducing phase margin. This fundamentally limits the achievable crossover frequency:

f_c,max < f_z / 5 to f_z / 3 ≈ 667 Hz to 1,111 Hz

For this design, targeting f_c ≈ 800 Hz provides adequate margin from the RHP zero.

## 3.4 Line-to-Output Transfer Function (G_vg)

G_vg(s) = G_g0 × 1 / (1 + s/(Qω_0) + s²/ω_0²)

Where: G_g0 = 1/D' = 2.5 (≈ 8 dB)

Note: G_vg has no RHP zero, only the LC resonance.

## 3.5 Output Impedance (Z_out)

Z_out(s) = Z_0 × (sL/D'²) / (1 + s/(Qω_0) + s²/ω_0²)

Where: Z_0 = sL/D'² at low frequencies, then resonant peak, then parallel RC

## 3.6 Summary of Small-Signal Parameters

| Parameter | Value |
| --- | --- |
| DC Gain (G_d0) | 300 V (49.5 dB) |
| Resonant Frequency (f_0) | 1,358 Hz |
| Quality Factor (Q) | 7.77 (17.8 dB peaking) |
| RHP Zero Frequency (f_z) | 3,333 Hz |
| Line-to-Output DC Gain (G_g0) | 2.5 (8 dB) |

# 4. Step 3: Bode Plot Analysis

## 4.1 Uncompensated Loop Gain

The uncompensated loop gain is:

T_u(s) = G_vd(s) × H(s) × (1/V_M)

Where H(s) is the voltage sensing network and 1/V_M is the PWM modulator gain.

## 4.2 G_vd Bode Plot Characteristics

The control-to-output transfer function exhibits the following behavior:

- DC to f_0: Flat at G_d0 = 49.5 dB, phase ≈ 0°

- At f_0 = 1,358 Hz: Resonant peak of +17.8 dB (Q = 7.77), phase drops through -90°

- f_0 to f_z: -40 dB/decade slope, phase approaching -180°

- At f_z = 3,333 Hz: RHP zero adds +20 dB/decade (net -20 dB/decade), but phase drops further toward -270°

- Above f_z: -20 dB/decade, phase settles at -270°

## 4.3 Modulator and Sensor Gains

PWM Modulator gain:

1/V_M = 1/3.3 = 0.303 (-10.4 dB)

Voltage sensing gain (to be designed):

H = V_ref / V = 2.5 / 120 = 0.0208 (-33.6 dB)

## 4.4 Critical Frequencies for Compensator Design

| Frequency | Value & Significance |
| --- | --- |
| f_0 (LC resonance) | 1,358 Hz - Compensator zeros here |
| f_z (RHP zero) | 3,333 Hz - Must cross BELOW this |
| f_c (target crossover) | 800 Hz - Safe margin from f_z |
| f_s/10 (upper limit) | 15,000 Hz - Rule of thumb max |

# 5. Step 4: Compensator Design

## 5.1 Compensator Selection

Due to the RHP zero and LC resonance, a Type III (PID) compensator is required:

G_c(s) = G_c0 × (1 + s/ω_z1)(1 + s/ω_z2) / [s(1 + s/ω_p1)(1 + s/ω_p2)]

The compensator must:

- Provide integrator for zero steady-state error

- Place zeros at or below f_0 to compensate LC resonance

- Achieve crossover well below the RHP zero

- Provide adequate phase boost at crossover

## 5.2 Design Strategy

Given the RHP zero constraint, the design strategy differs from buck converter:

- Target crossover: f_c = 800 Hz (< f_z/4)

- Place compensator zeros at f_0/2 = 679 Hz to provide phase boost

- Place compensator poles above f_c but below f_z/2

- Required phase margin: 45° minimum (60° target)

## 5.3 Compensator Parameters

Compensator zeros (both at same frequency for simplicity):

f_z1 = f_z2 = f_0 / 2 = 679 Hz

ω_z1 = ω_z2 = 4,266 rad/s

Compensator poles:

f_p1 = f_z × 0.8 = 2,666 Hz (just below RHP zero)

f_p2 = f_s / 3 = 50,000 Hz (high-frequency noise rejection)

Compensator DC gain calculation:

At f_c = 800 Hz, the uncompensated loop gain magnitude is approximately:

|T_u(j2πf_c)| ≈ G_d0 × H × (1/V_M) × (f_0/f_c)² × (1/Q)

|T_u| ≈ 300 × 0.0208 × 0.303 × (1358/800)² × (1/7.77) = 0.70 (-3 dB)

Required compensator gain at f_c to achieve 0 dB crossover:

|G_c(j2πf_c)| = 1/0.70 = 1.43 (3 dB)

## 5.4 Phase Margin Verification

At crossover frequency f_c = 800 Hz:

- Plant phase (G_vd): ≈ -120° (resonance effect)

- RHP zero contribution: ≈ -14° (800/3333 × 57.3°)

- Modulator phase: 0°

- Compensator integrator: -90°

- Compensator zeros: +2 × arctan(800/679) = +2 × 49.7° = +99.4°

- Compensator poles: -arctan(800/2666) - arctan(800/50000) = -17° - 0.9° = -17.9°

Total phase at crossover:

φ = -180° + (-120° - 14° - 90° + 99.4° - 17.9°) = -180° + (-142.5°) → Phase margin ≈ 37.5°

To achieve 45° phase margin, adjust zeros to lower frequency:

f_z1 = f_z2 = 500 Hz (provides additional phase boost)

# 6. Step 5: Loop Gain Verification

## 6.1 Complete Loop Gain Transfer Function

T(s) = G_c(s) × (1/V_M) × G_vd(s) × H(s)

## 6.2 MATLAB Verification Code

The following MATLAB code generates Bode plots for verification:

% Boost Converter Transfer Function Parameters
Vg = 48; V = 120; L = 220e-6; C = 100e-6;
R = 28.8; Vm = 3.3; Vref = 2.5; fs = 150e3;
D = 1 - Vg/V;  Dprime = 1 - D;
Gd0 = V/Dprime;
w0 = Dprime/sqrt(L*C);
Q = Dprime*R*sqrt(C/L);
wz_rhp = Dprime^2*R/L;  % RHP zero!
% Control-to-output (note negative sign for RHP zero)
s = tf('s');
Gvd = Gd0 * (1 - s/wz_rhp) / (1 + s/(Q*w0) + s^2/w0^2);
% Compensator
Gc0 = 1.5;  wz = 2*pi*500;  wp1 = 2*pi*2666;  wp2 = 2*pi*50000;
Gc = Gc0 * (1 + s/wz)^2 / (s * (1 + s/wp1) * (1 + s/wp2));
% Sensor and Modulator
H = Vref/V;  Fm = 1/Vm;
% Loop gain
T = Gc * Fm * Gvd * H;
bode(T); margin(T);

## 6.3 Expected Loop Gain Characteristics

| Parameter | Target Value |
| --- | --- |
| Crossover Frequency | ~800 Hz |
| Phase Margin | >45° |
| Gain Margin | >10 dB |
| Low-frequency gain | High (integrator) |

# 7. Step 6: Op-Amp Implementation

## 7.1 Type III Compensator Circuit

The Type III compensator is implemented using a single op-amp with the following topology:

- Feedback network: R2-C2 in series, paralleled with C1

- Input network: R1 in series with R3-C3 parallel combination

## 7.2 Transfer Function

G_c(s) = (1 + sR2C2)(1 + sR3C3) / [sR1(C1 + C2)(1 + sR2C1C2/(C1+C2))(1 + sR3C3R1/(R1+R3))]

## 7.3 Component Value Calculation

Using the designed pole/zero frequencies:

Step 1: Choose C2 (typically 1 nF to 100 nF):

C2 = 10 nF

Step 2: Calculate R2 from zero frequency:

R2 = 1/(2π × f_z × C2) = 1/(2π × 500 × 10nF) = 31.8 kΩ → 33 kΩ

Step 3: Calculate C1 from first pole:

C1 = 1/(2π × f_p1 × R2) = 1/(2π × 2666 × 33kΩ) = 1.81 nF → 1.8 nF

Step 4: Calculate R1 for desired gain:

R1 = R2 × C2 / (G_c0 × (C1 + C2)) = 33k × 10n / (1.5 × 11.8n) = 18.6 kΩ → 18 kΩ

Step 5: Calculate R3 and C3 for second zero:

C3 = 10 nF (same as C2 for convenience)

R3 = 1/(2π × f_z × C3) = 33 kΩ

## 7.4 Component Summary

| Component | Calculated | Standard Value |
| --- | --- | --- |
| R1 | 18.6 kΩ | 18 kΩ |
| R2 | 31.8 kΩ | 33 kΩ |
| R3 | 31.8 kΩ | 33 kΩ |
| C1 | 1.81 nF | 1.8 nF |
| C2 | 10 nF | 10 nF |
| C3 | 10 nF | 10 nF |

## 7.5 Voltage Sensing Network

Resistive divider to scale output voltage to reference level:

H = R_b / (R_a + R_b) = V_ref / V = 2.5 / 120 = 0.0208

Choose R_b = 2.1 kΩ, then R_a = 98.9 kΩ (use 100 kΩ)

# 8. Step 7: Complete Circuit Schematic

## 8.1 System Block Diagram

The complete voltage-mode control system consists of:

- Synchronous boost power stage (L, Q1, Q2, C)

- Voltage sensing resistor divider (R_a, R_b)

- Error amplifier (compares V_sense to V_ref)

- Type III compensator (provides loop shaping)

- PWM modulator (converts control voltage to duty cycle)

- Gate drivers (interface to power MOSFETs)

## 8.2 Synchronous Boost Power Stage

Key component specifications for 98%+ efficiency:

| Component | Specification | Suggested Part |
| --- | --- | --- |
| Q1 (Low-side) | 100V, 8mΩ, SiC | C3M0065100K |
| Q2 (High-side) | 200V, 15mΩ, SiC | C3M0075120K |
| L1 | 220µH, 15A, 15mΩ | SER2918H-224 |
| C_out | 100µF, 200V, Film | B32774D2107 |
| Gate Driver | Isolated, 4A | UCC21520 |

## 8.3 Control Circuit Components

PWM Controller: Use integrated controller with adjustable V_M or discrete comparator approach

Op-Amp: Low-noise, rail-to-rail (e.g., OPA2192)

Reference: 2.5V precision reference (e.g., REF3025)

# 9. Step 8: Simulation Verification

## 9.1 Simulation Test Cases

The closed-loop system must be verified under:

- Test 1: Input voltage step from 48V to 60V

- Test 2: Input voltage step from 48V to 25V

- Test 3: Load current step from 4.17A to 2A

- Test 4: Load current step from 4.17A to 5A

## 9.2 Performance Requirements

| Metric | Requirement |
| --- | --- |
| Output voltage regulation | ±1% (118.8V - 121.2V) |
| Transient deviation | <5% for step changes |
| Settling time | <5 ms to within 1% |
| No sustained oscillation | Stable under all conditions |

## 9.3 Simulink Model Structure

The time-domain simulation should include:

- Switching model of synchronous boost (not averaged)

- Realistic component parasitics (ESR, DCR)

- Gate driver delays

- Complete feedback loop with discrete compensator

## 9.4 Expected Transient Behavior

Due to the RHP zero, the boost converter exhibits non-minimum phase response:

- For a step increase in duty cycle, output initially DECREASES before increasing

- This is fundamentally different from buck converter behavior

- The RHP zero limits how aggressively the controller can respond

# 10. Key Differences: Buck vs. Boost Control Design

## 10.1 Transfer Function Comparison

| Aspect | Buck Converter | Boost Converter |
| --- | --- | --- |
| G_vd DC Gain | V_g | V/D' (varies with D) |
| RHP Zero | None | Yes: ω_z = D'²R/L |
| Resonant freq | 1/√(LC) | D'/√(LC) |
| Q factor | R√(C/L) | D'R√(C/L) |
| Max f_c | f_s/10 | min(f_s/10, f_z/3) |

## 10.2 Design Implications

The RHP zero fundamentally limits boost converter control bandwidth:

- Buck converter: f_c limited only by switching frequency (f_s/10)

- Boost converter: f_c limited by BOTH f_s/10 AND RHP zero frequency

- At high duty cycles, RHP zero frequency decreases, making control harder

- Wide input range designs must account for worst-case RHP zero location

## 10.3 Compensator Design Differences

- Buck: Can use aggressive Type II or Type III, high crossover achievable

- Boost: Must use Type III, crossover limited by RHP zero

- Boost requires more conservative design with larger phase margins

- Boost transient response inherently slower due to bandwidth limitation

## 10.4 Application to MPPT

For the Project Lighthouse MPPT application:

- The boost topology is required to step up solar panel voltage (25-60V) to bus voltage (120V)

- The RHP zero constraint means the MPPT algorithm should perturb slowly

- Perturb & Observe step size must account for the control loop settling time

- Incremental Conductance can be more aggressive since it predicts MPP direction

# Appendix: Complete Design Parameters

## A.1 Power Stage Parameters

| Parameter | Value |
| --- | --- |
| Input Voltage (nominal) | 48 V |
| Output Voltage | 120 V |
| Output Power | 500 W |
| Duty Cycle (nominal) | 0.60 |
| D' (nominal) | 0.40 |
| Inductance | 220 µH |
| Output Capacitance | 100 µF |
| Load Resistance | 28.8 Ω |
| Switching Frequency | 150 kHz |

## A.2 Small-Signal Parameters

| Parameter | Value |
| --- | --- |
| G_d0 | 300 V (49.5 dB) |
| f_0 | 1,358 Hz |
| Q | 7.77 |
| f_z (RHP) | 3,333 Hz |
| G_g0 | 2.5 (8 dB) |

## A.3 Compensator Parameters

| Parameter | Value |
| --- | --- |
| Compensator Type | Type III (PID) |
| Zero Frequency (both) | 500 Hz |
| Pole 1 Frequency | 2,666 Hz |
| Pole 2 Frequency | 50,000 Hz |
| Target Crossover | 800 Hz |
| Target Phase Margin | >45° |

## A.4 Op-Amp Component Values

| Component | Value |
| --- | --- |
| R1 | 18 kΩ |
| R2 | 33 kΩ |
| R3 | 33 kΩ |
| C1 | 1.8 nF |
| C2 | 10 nF |
| C3 | 10 nF |
| R_a (sensing) | 100 kΩ |
| R_b (sensing) | 2.1 kΩ |

--- End of Design Document ---