Synchronous Boost Converter

Hardware Implementation Guide

STM32 Firmware & Power Stage Design

For Solar MPPT Applications

500W | 25-60V Input | 120V Output

Part 2: Synchronous Switching Implementation

(Companion to Feedback Control Design Document)

# Table of Contents

Part A: Power Stage Hardware

1. Synchronous Boost Converter Fundamentals

2. Switching Frequency Selection

3. Component Selection & Sizing

4. Gate Driver Design & Dead-Time

5. Protection Circuits

Part B: STM32 Firmware Implementation

6. Timer Configuration for Synchronous PWM

7. ADC Configuration for Sensing

8. Main Control Loop Structure

9. Protection Implementation

10. MPPT Algorithm Integration

Part C: Testing & Validation

11. Bench Testing Procedures

12. Oscilloscope Verification

13. Efficiency Measurement

PART A: POWER STAGE HARDWARE

# 1. Synchronous Boost Converter Fundamentals

## 1.1 Why Synchronous Rectification?

In a conventional boost converter, a diode conducts during the OFF interval (D' period). The diode forward voltage drop V_D (typically 0.5-1.5V for silicon, 0.3-0.5V for Schottky) causes significant power loss:

P_diode = V_D × I_avg × D' = V_D × I_out

For our 500W, 120V output converter at 4.17A output current with a 0.5V Schottky:

P_diode = 0.5V × 4.17A = 2.08W

A synchronous MOSFET with R_DS(on) = 15mΩ conducting the same current:

P_MOSFET = I²_rms × R_DS(on) ≈ (10.4A)² × 0.015Ω = 1.62W

The synchronous approach saves ~0.5W and becomes even more advantageous at higher currents.

## 1.2 Synchronous Boost Topology

The synchronous boost converter uses two MOSFETs:

- Q1 (Low-Side): Main switch, conducts during ON interval (0 to DTs)

- Q2 (High-Side): Synchronous rectifier, conducts during OFF interval (DTs to Ts)

Circuit operation:

- Interval 1 (0 < t < DTs): Q1 ON, Q2 OFF - Inductor charges from input

- Interval 2 (DTs < t < Ts): Q1 OFF, Q2 ON - Inductor delivers energy to output

## 1.3 Critical Timing Requirement: Dead-Time

NEVER allow Q1 and Q2 to conduct simultaneously! This creates a shoot-through condition:

- Direct short from output to ground through both MOSFETs

- Extremely high current spike (limited only by R_DS(on))

- Potential MOSFET destruction within microseconds

Dead-time is a brief period where BOTH switches are OFF during transitions:

- Q1 turn-off to Q2 turn-on: Dead-time 1

- Q2 turn-off to Q1 turn-on: Dead-time 2

During dead-time, current flows through the body diode of the OFF switch.

# 2. Switching Frequency Selection

## 2.1 The Fundamental Tradeoff

Switching frequency selection involves balancing competing factors:

| Higher Frequency Benefits | Higher Frequency Drawbacks |
| --- | --- |
| Smaller inductor (L ∝ 1/fs) | Higher switching losses (P_sw ∝ fs) |
| Smaller capacitor for same ripple | Higher gate drive losses (P_gate ∝ fs) |
| Faster transient response | More EMI/noise |
| Smaller physical size | Harder timing requirements |

## 2.2 Loss Analysis for Frequency Selection

### 2.2.1 Conduction Losses (Independent of fs)

P_cond = I²_L,rms × (R_DS,Q1 × D + R_DS,Q2 × D') + I²_L,rms × R_L

For our design with I_L ≈ 10.4A, R_DS = 15mΩ each, R_L = 15mΩ, D = 0.6:

P_cond = (10.4)² × (0.015 × 0.6 + 0.015 × 0.4) + (10.4)² × 0.015

P_cond = 108 × 0.015 + 108 × 0.015 = 1.62W + 1.62W = 3.24W

### 2.2.2 Switching Losses (Proportional to fs)

For hard-switched converters, switching loss per transition:

E_sw = 0.5 × V × I × (t_rise + t_fall)

With V = 120V (output voltage seen during switching), I = 10.4A, t_r + t_f ≈ 40ns typical:

E_sw = 0.5 × 120 × 10.4 × 40ns = 25 µJ per transition

P_sw = E_sw × 2 × fs = 50µJ × fs

| fs (kHz) | P_sw (W) | L_min (µH) | Total Loss Est. |
| --- | --- | --- | --- |
| 50 | 2.5 | 576 | 5.7W |
| 100 | 5.0 | 288 | 8.2W |
| 150 | 7.5 | 192 | 10.7W |
| 200 | 10.0 | 144 | 13.2W |

## 2.3 Recommended Frequency: 100 kHz

For your 500W solar MPPT application, 100 kHz provides:

- Reasonable inductor size (~220-330 µH practical)

- Manageable switching losses (~5W)

- Compatible with standard SiC/GaN MOSFETs

- STM32 timer resolution adequate (10µs period = 1000 counts at 100MHz)

- Control loop bandwidth limited by RHP zero anyway (~800 Hz max)

## 2.4 Testing Different Frequencies

To experimentally determine optimal frequency:

- Step 1: Build the converter with adjustable fs (change timer period)

- Step 2: Operate at fixed input/output/load

- Step 3: Measure efficiency at 50, 75, 100, 125, 150 kHz

- Step 4: Plot efficiency vs. frequency - find the peak

- Step 5: Check thermal performance at that frequency

# 3. Component Selection & Sizing

## 3.1 Inductor Design

### 3.1.1 Minimum Inductance for CCM

To ensure CCM operation at minimum load (say 10% = 50W):

L_min = V_g × D × (1-D) / (2 × I_out,min × fs)

At worst case (V_g = 60V, D = 0.5, I_out,min = 0.42A, fs = 100kHz):

L_min = 60 × 0.5 × 0.5 / (2 × 0.42 × 100k) = 178 µH

### 3.1.2 Inductance for Target Ripple

For 20% peak-to-peak current ripple at full load:

ΔI_L = V_g × D / (L × fs)

L = V_g × D / (ΔI_L × fs) = 48 × 0.6 / (2.08 × 100k) = 138 µH

Select L = 220 µH for margin (standard value, keeps ripple < 15%).

### 3.1.3 Inductor Specifications

| Parameter | Requirement |
| --- | --- |
| Inductance | 220 µH ± 20% |
| Saturation Current | > 15A (I_L,peak) |
| RMS Current Rating | > 12A |
| DC Resistance | < 20 mΩ |
| Suggested Part | Coilcraft XAL1010-224ME |

## 3.2 MOSFET Selection

### 3.2.1 Voltage Rating

MOSFETs see V_out + margin. For 120V output:

V_DS,rated > 1.5 × V_out = 180V minimum

Select 200V rated MOSFETs for adequate margin.

### 3.2.2 Current Rating

RMS current through each MOSFET:

I_Q1,rms = I_L × √D = 10.4 × √0.6 = 8.06A

I_Q2,rms = I_L × √D' = 10.4 × √0.4 = 6.58A

Select MOSFETs rated for > 20A continuous for thermal margin.

### 3.2.3 Recommended MOSFETs

| Parameter | Si MOSFET | SiC MOSFET | GaN FET |
| --- | --- | --- | --- |
| Example Part | IPP200N25N3 | C3M0065100K | GS-065-030-2-L |
| R_DS(on) | 25 mΩ | 65 mΩ | 30 mΩ |
| Cost | $2-5 | $8-15 | $10-20 |
| Best for | < 100kHz | 100-500kHz | > 200kHz |

## 3.3 Output Capacitor

### 3.3.1 Capacitance for Ripple

ΔV_out = I_out × D / (C × fs)

For ΔV < 1% of 120V = 1.2V:

C > I_out × D / (ΔV × fs) = 4.17 × 0.6 / (1.2 × 100k) = 20.8 µF

Select C = 100 µF for margin and transient performance.

### 3.3.2 Capacitor Specifications

| Parameter | Requirement |
| --- | --- |
| Capacitance | 100 µF |
| Voltage Rating | > 150V (use 200V) |
| ESR | < 50 mΩ |
| Ripple Current | > 5A RMS |
| Type | Film or Low-ESR Electrolytic |

# 4. Gate Driver Design & Dead-Time

## 4.1 Gate Driver Requirements

The synchronous boost needs a half-bridge gate driver:

- High-side driver for Q2 (floating, bootstrap or isolated)

- Low-side driver for Q1 (ground-referenced)

- Built-in dead-time generation OR accept external PWM with dead-time

- Fast rise/fall times (< 50ns) for efficient switching

## 4.2 Recommended Gate Drivers

| Part Number | Type | Dead-Time | Peak Current |
| --- | --- | --- | --- |
| IR2104 | Bootstrap | 520ns fixed | 0.36A/0.6A |
| IR2110 | Bootstrap | None (external) | 2A/2A |
| UCC21520 | Isolated | Programmable | 4A/6A |
| Si8274 | Isolated | None (external) | 4A/4A |

## 4.3 Dead-Time Calculation

### 4.3.1 Required Dead-Time Components

Total dead-time must exceed:

t_dead > t_off(Q_turning_off) + t_prop(driver) + margin

Typical MOSFET turn-off time: 20-50ns

Gate driver propagation delay: 20-100ns

Safety margin: 50-100ns

t_dead,min = 50ns + 50ns + 100ns = 200ns

### 4.3.2 Maximum Dead-Time

Dead-time too long causes:

- Body diode conduction losses (V_D ≈ 0.7-1.5V during dead-time)

- Reverse recovery losses when body diode turns off

t_dead,max < 500ns recommended for efficiency

### 4.3.3 Recommended Dead-Time: 200-300ns

For STM32 at 100MHz timer clock:

Dead-time counts = t_dead × f_timer = 300ns × 100MHz = 30 counts

## 4.4 Bootstrap Capacitor Sizing

For bootstrap gate drivers (IR2104, IR2110):

C_boot > Q_gate × 10 / V_cc = (50nC × 10) / 12V = 42nF

Use C_boot = 100nF ceramic (standard practice).

Bootstrap refresh requirement:

- Q2 must turn ON periodically to refresh bootstrap capacitor

- Maximum D < 95% for reliable bootstrap operation

- At D = 0.792 (V_g = 25V), this is satisfied

# 5. Protection Circuits

## 5.1 Overcurrent Protection

### 5.1.1 Cycle-by-Cycle Current Limiting

Sense inductor current and terminate PWM pulse if threshold exceeded:

- Current sense resistor in series with low-side MOSFET source

- Or use MOSFET R_DS(on) sensing

- Or hall-effect current sensor (ACS712, ACS723)

Implementation:

- Compare I_sense to I_limit threshold

- Feed comparator output to timer BREAK input

- Timer immediately turns off PWM outputs

- Fault cleared on next PWM cycle (soft restart)

### 5.1.2 Current Sense Resistor Design

For R_sense in low-side MOSFET source:

V_sense = I_L × R_sense

At I_limit = 15A, want V_sense ≈ 0.3V for comparator:

R_sense = 0.3V / 15A = 20 mΩ

P_sense = I²_rms × R_sense = (10.4)² × 0.02 = 2.16W

Use a 3W or larger resistor, or parallel multiple smaller ones.

## 5.2 Overvoltage Protection

### 5.2.1 Output OVP

If V_out exceeds threshold (e.g., 135V = 112.5% of nominal):

- Reduce duty cycle immediately

- Or shut down converter entirely

- Comparator monitors output via resistor divider

### 5.2.2 Input Undervoltage Lockout (UVLO)

Prevent operation below minimum input voltage:

- Solar panel V_mpp can vary; ensure V_g > V_g,min = 25V

- Below this, D becomes too high (> 0.8), stressing components

## 5.3 Thermal Protection

Monitor MOSFET temperature with NTC thermistor:

- Warning threshold: 80°C - reduce power

- Shutdown threshold: 100°C - disable converter

- Hysteresis: 10°C to prevent oscillation

## 5.4 Soft-Start

Ramp up duty cycle gradually at startup:

- Prevents inrush current surge into output capacitor

- Allows control loop to stabilize

- Typical soft-start time: 10-50ms

Implementation:

D(t) = D_target × (t / t_softstart) for t < t_softstart

PART B: STM32 FIRMWARE IMPLEMENTATION

# 6. Timer Configuration for Synchronous PWM

## 6.1 STM32 Timer Features for Motor/Power Control

Use TIM1 or TIM8 (Advanced Control Timers) because they have:

- Complementary outputs (CH1/CH1N, CH2/CH2N, etc.)

- Programmable dead-time generator (DTG register)

- Break input for fault protection

- Center-aligned mode for lower ripple

## 6.2 Timer Configuration Code

The following code configures TIM1 for synchronous boost PWM:

/*******************************************************************************

* File: sync_boost_pwm.c

* Description: STM32 Timer configuration for synchronous boost converter

* Target: STM32F4xx / STM32G4xx (adjust for your specific MCU)

******************************************************************************/

#include "stm32f4xx_hal.h"

/* ============== CONFIGURATION PARAMETERS ============== */

#define TIMER_CLOCK_HZ      100000000UL  // Timer input clock (APB2 for TIM1)

#define SWITCHING_FREQ_HZ   100000UL     // 100 kHz switching frequency

#define DEAD_TIME_NS        300          // 300ns dead-time

#define PWM_PERIOD          (TIMER_CLOCK_HZ / SWITCHING_FREQ_HZ)  // = 1000 counts

/* Dead-time calculation for STM32:

* DTG[7:0] encodes dead-time. For DTG < 128:

* t_dead = DTG × t_DTS where t_DTS = 1/TIMER_CLOCK

* DTG = t_dead × TIMER_CLOCK = 300ns × 100MHz = 30 */

#define DEAD_TIME_COUNTS    30

TIM_HandleTypeDef htim1;

/*******************************************************************************

* Function: SyncBoost_PWM_Init

* Description: Initialize TIM1 for complementary PWM with dead-time

*

* Pin Mapping (STM32F4):

*   PA8  = TIM1_CH1  = Q1 Gate (Low-side MOSFET)

*   PA7  = TIM1_CH1N = Q2 Gate (High-side MOSFET) - ACTIVE LOW!

*

* IMPORTANT: For boost converter, Q1 and Q2 are COMPLEMENTARY:

*   - When CH1 is HIGH, Q1 is ON (inductor charging)

*   - When CH1N is HIGH, Q2 is ON (energy transfer to output)

******************************************************************************/

HAL_StatusTypeDef SyncBoost_PWM_Init(void)

{

TIM_ClockConfigTypeDef sClockSourceConfig = {0};

TIM_MasterConfigTypeDef sMasterConfig = {0};

TIM_OC_InitTypeDef sConfigOC = {0};

TIM_BreakDeadTimeConfigTypeDef sBreakDeadTimeConfig = {0};

/* Enable TIM1 clock */

__HAL_RCC_TIM1_CLK_ENABLE();

/* Configure TIM1 base */

htim1.Instance = TIM1;

htim1.Init.Prescaler = 0;                    // No prescaler: full 100MHz

htim1.Init.CounterMode = TIM_COUNTERMODE_UP; // Up-counting mode

htim1.Init.Period = PWM_PERIOD - 1;          // 1000-1 = 999 for 100kHz

htim1.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim1.Init.RepetitionCounter = 0;

htim1.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_ENABLE;

if (HAL_TIM_PWM_Init(&htim1) != HAL_OK) {

return HAL_ERROR;

}

/* Configure PWM Channel 1 (Q1 - Low Side) */

sConfigOC.OCMode = TIM_OCMODE_PWM1;          // PWM Mode 1

sConfigOC.Pulse = 0;                         // Start at 0% duty (safe)

sConfigOC.OCPolarity = TIM_OCPOLARITY_HIGH;  // Active high

sConfigOC.OCNPolarity = TIM_OCNPOLARITY_HIGH;// Complementary also active high

sConfigOC.OCFastMode = TIM_OCFAST_DISABLE;

sConfigOC.OCIdleState = TIM_OCIDLESTATE_RESET;   // LOW when disabled

sConfigOC.OCNIdleState = TIM_OCNIDLESTATE_RESET; // LOW when disabled

if (HAL_TIM_PWM_ConfigChannel(&htim1, &sConfigOC, TIM_CHANNEL_1) != HAL_OK) {

return HAL_ERROR;

}

/* Configure Dead-Time and Break functionality */

sBreakDeadTimeConfig.OffStateRunMode = TIM_OSSR_DISABLE;

sBreakDeadTimeConfig.OffStateIDLEMode = TIM_OSSI_DISABLE;

sBreakDeadTimeConfig.LockLevel = TIM_LOCKLEVEL_OFF;

sBreakDeadTimeConfig.DeadTime = DEAD_TIME_COUNTS;  // 30 counts = 300ns

sBreakDeadTimeConfig.BreakState = TIM_BREAK_ENABLE;  // Enable fault input

sBreakDeadTimeConfig.BreakPolarity = TIM_BREAKPOLARITY_HIGH; // Fault on HIGH

sBreakDeadTimeConfig.AutomaticOutput = TIM_AUTOMATICOUTPUT_DISABLE;

if (HAL_TIMEx_ConfigBreakDeadTime(&htim1, &sBreakDeadTimeConfig) != HAL_OK) {

return HAL_ERROR;

}

return HAL_OK;

}

/*******************************************************************************

* Function: SyncBoost_PWM_Start

* Description: Start PWM outputs with complementary channels

******************************************************************************/

HAL_StatusTypeDef SyncBoost_PWM_Start(void)

{

/* Start complementary PWM on Channel 1 and Channel 1N */

if (HAL_TIM_PWM_Start(&htim1, TIM_CHANNEL_1) != HAL_OK) {

return HAL_ERROR;

}

if (HAL_TIMEx_PWMN_Start(&htim1, TIM_CHANNEL_1) != HAL_OK) {

return HAL_ERROR;

}

/* Enable main output (MOE bit) - CRITICAL for advanced timers! */

__HAL_TIM_MOE_ENABLE(&htim1);

return HAL_OK;

}

/*******************************************************************************

* Function: SyncBoost_PWM_Stop

* Description: Stop all PWM outputs (emergency shutdown)

******************************************************************************/

void SyncBoost_PWM_Stop(void)

{

/* Disable main output - both MOSFETs turn OFF immediately */

__HAL_TIM_MOE_DISABLE(&htim1);

HAL_TIM_PWM_Stop(&htim1, TIM_CHANNEL_1);

HAL_TIMEx_PWMN_Stop(&htim1, TIM_CHANNEL_1);

}

/*******************************************************************************

* Function: SyncBoost_SetDuty

* Description: Set duty cycle for the boost converter

*

* @param duty_percent: Duty cycle 0.0 to 0.95 (max 95% for bootstrap refresh)

*                      D = duty_percent corresponds to Q1 ON time

*

* TIMING DIAGRAM:

*        |<------ Ts = 10µs (100kHz) ------>|

*        |<--- D×Ts --->|<----- D'×Ts ----->|

* Q1:    |######HIGH#####|_______LOW________|

* Q2:    |______LOW______|######HIGH########|

*        |   ^dead^      |   ^dead^         |

*        |   time        |   time           |

******************************************************************************/

void SyncBoost_SetDuty(float duty_percent)

{

/* Clamp duty cycle for safety */

if (duty_percent < 0.0f) duty_percent = 0.0f;

if (duty_percent > 0.95f) duty_percent = 0.95f;  // Max 95% for bootstrap

/* Calculate compare value */

uint32_t compare = (uint32_t)(duty_percent * (float)PWM_PERIOD);

/* Update CCR1 register (with preload, takes effect next period) */

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, compare);

}

## 6.3 GPIO Configuration for PWM Outputs

/*******************************************************************************

* Function: SyncBoost_GPIO_Init

* Description: Configure GPIO pins for TIM1 PWM outputs

******************************************************************************/

void SyncBoost_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

__HAL_RCC_GPIOA_CLK_ENABLE();

/* PA8 = TIM1_CH1 (Q1 gate - Low side) */

/* PA7 = TIM1_CH1N (Q2 gate - High side) */

GPIO_InitStruct.Pin = GPIO_PIN_7 | GPIO_PIN_8;

GPIO_InitStruct.Mode = GPIO_MODE_AF_PP;        // Alternate function push-pull

GPIO_InitStruct.Pull = GPIO_PULLDOWN;          // Pull down when disabled

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_HIGH;  // Fast switching

GPIO_InitStruct.Alternate = GPIO_AF1_TIM1;     // AF1 = TIM1

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

}

# 7. ADC Configuration for Sensing

## 7.1 Signals to Measure

| Signal | Range | Scaling | Purpose |
| --- | --- | --- | --- |
| V_in (Solar) | 25-60V | ÷20 → 1.25-3V | MPPT input |
| V_out | 0-140V | ÷50 → 0-2.8V | Regulation |
| I_in (Solar) | 0-15A | ×0.02Ω → 0-0.3V | MPPT, OCP |
| Temperature | 0-150°C | NTC divider | Thermal prot. |

## 7.2 ADC Configuration Code

/*******************************************************************************

* File: sync_boost_adc.c

* Description: ADC configuration for boost converter sensing

******************************************************************************/

#include "stm32f4xx_hal.h"

/* ADC Channel assignments (adjust for your board) */

#define ADC_CHANNEL_VIN     ADC_CHANNEL_0   // PA0

#define ADC_CHANNEL_VOUT    ADC_CHANNEL_1   // PA1

#define ADC_CHANNEL_IIN     ADC_CHANNEL_2   // PA2

#define ADC_CHANNEL_TEMP    ADC_CHANNEL_3   // PA3

/* Scaling factors (adjust based on your voltage dividers) */

#define VIN_SCALE           20.0f    // V_actual = ADC_voltage × 20

#define VOUT_SCALE          50.0f    // V_actual = ADC_voltage × 50

#define IIN_SCALE           50.0f    // I_actual = ADC_voltage × 50 (1/R_sense)

#define ADC_VREF            3.3f     // ADC reference voltage

#define ADC_RESOLUTION      4096.0f  // 12-bit ADC

ADC_HandleTypeDef hadc1;

volatile uint16_t adc_raw[4];  // DMA buffer for 4 channels

/*******************************************************************************

* Function: SyncBoost_ADC_Init

* Description: Configure ADC1 with DMA for continuous multi-channel scanning

******************************************************************************/

HAL_StatusTypeDef SyncBoost_ADC_Init(void)

{

ADC_ChannelConfTypeDef sConfig = {0};

__HAL_RCC_ADC1_CLK_ENABLE();

hadc1.Instance = ADC1;

hadc1.Init.ClockPrescaler = ADC_CLOCK_SYNC_PCLK_DIV4;

hadc1.Init.Resolution = ADC_RESOLUTION_12B;

hadc1.Init.ScanConvMode = ENABLE;            // Scan multiple channels

hadc1.Init.ContinuousConvMode = ENABLE;      // Continuous conversion

hadc1.Init.DiscontinuousConvMode = DISABLE;

hadc1.Init.ExternalTrigConvEdge = ADC_EXTERNALTRIGCONVEDGE_NONE;

hadc1.Init.ExternalTrigConv = ADC_SOFTWARE_START;

hadc1.Init.DataAlign = ADC_DATAALIGN_RIGHT;

hadc1.Init.NbrOfConversion = 4;              // 4 channels

hadc1.Init.DMAContinuousRequests = ENABLE;

hadc1.Init.EOCSelection = ADC_EOC_SEQ_CONV;

if (HAL_ADC_Init(&hadc1) != HAL_OK) return HAL_ERROR;

/* Configure each channel */

sConfig.SamplingTime = ADC_SAMPLETIME_84CYCLES;  // ~2µs per sample

sConfig.Channel = ADC_CHANNEL_VIN;

sConfig.Rank = 1;

HAL_ADC_ConfigChannel(&hadc1, &sConfig);

sConfig.Channel = ADC_CHANNEL_VOUT;

sConfig.Rank = 2;

HAL_ADC_ConfigChannel(&hadc1, &sConfig);

sConfig.Channel = ADC_CHANNEL_IIN;

sConfig.Rank = 3;

HAL_ADC_ConfigChannel(&hadc1, &sConfig);

sConfig.Channel = ADC_CHANNEL_TEMP;

sConfig.Rank = 4;

HAL_ADC_ConfigChannel(&hadc1, &sConfig);

return HAL_OK;

}

/*******************************************************************************

* Function: SyncBoost_GetMeasurements

* Description: Convert raw ADC values to engineering units

*

* @param v_in:  Pointer to store input voltage (V)

* @param v_out: Pointer to store output voltage (V)

* @param i_in:  Pointer to store input current (A)

* @param temp:  Pointer to store temperature (°C)

******************************************************************************/

void SyncBoost_GetMeasurements(float *v_in, float *v_out, float *i_in, float *temp)

{

/* Convert raw ADC to voltage (0-3.3V) then scale */

float adc_vin  = (float)adc_raw[0] * ADC_VREF / ADC_RESOLUTION;

float adc_vout = (float)adc_raw[1] * ADC_VREF / ADC_RESOLUTION;

float adc_iin  = (float)adc_raw[2] * ADC_VREF / ADC_RESOLUTION;

float adc_temp = (float)adc_raw[3] * ADC_VREF / ADC_RESOLUTION;

/* Apply scaling factors */

*v_in  = adc_vin * VIN_SCALE;

*v_out = adc_vout * VOUT_SCALE;

*i_in  = adc_iin * IIN_SCALE;

/* Temperature from NTC (simplified - use lookup table for accuracy) */

/* Assumes 10k NTC with 10k pullup to 3.3V */

float r_ntc = 10000.0f * adc_temp / (ADC_VREF - adc_temp);

*temp = 1.0f / (1.0f/298.15f + log(r_ntc/10000.0f)/3950.0f) - 273.15f;

}

# 8. Main Control Loop Structure

## 8.1 Control Loop Overview

The control system has two nested loops:

- Outer Loop (MPPT): Runs every 10-100ms, adjusts V_ref to track maximum power point

- Inner Loop (Voltage Regulation): Runs every 10-100µs, adjusts duty cycle to maintain V_out = 120V

## 8.2 Main Control Code

/*******************************************************************************

* File: sync_boost_control.c

* Description: Main control loop for synchronous boost MPPT converter

******************************************************************************/

#include "stm32f4xx_hal.h"

#include "sync_boost_pwm.h"

#include "sync_boost_adc.h"

/* ============== CONTROL PARAMETERS ============== */

#define VOUT_TARGET         120.0f   // Target output voltage

#define VOUT_MAX            135.0f   // OVP threshold

#define IIN_MAX             15.0f    // OCP threshold

#define TEMP_MAX            100.0f   // Thermal shutdown

#define TEMP_WARN           80.0f    // Thermal warning (reduce power)

/* PI Controller gains (tune experimentally) */

#define KP_VOLTAGE          0.001f   // Proportional gain

#define KI_VOLTAGE          0.05f    // Integral gain

/* MPPT parameters */

#define MPPT_STEP           0.5f     // Voltage step for P&O (V)

#define MPPT_PERIOD_MS      50       // MPPT update period (ms)

/* Control state variables */

typedef struct {

float duty;              // Current duty cycle (0-0.95)

float v_in;              // Input voltage (V)

float v_out;             // Output voltage (V)

float i_in;              // Input current (A)

float temp;              // Temperature (°C)

float power_in;          // Input power (W)

float integral_error;    // PI controller integral term

float v_ref_mppt;        // MPPT voltage reference

float power_prev;        // Previous power for P&O

int8_t mppt_direction;   // P&O direction (+1 or -1)

uint8_t fault_flags;     // Fault status bits

uint8_t soft_start_done; // Soft-start complete flag

} BoostControl_t;

BoostControl_t ctrl = {0};

/*******************************************************************************

* Function: SyncBoost_ControlInit

* Description: Initialize control system

******************************************************************************/

void SyncBoost_ControlInit(void)

{

ctrl.duty = 0.0f;           // Start at 0% duty

ctrl.integral_error = 0.0f;

ctrl.v_ref_mppt = 40.0f;    // Initial MPPT voltage guess

ctrl.power_prev = 0.0f;

ctrl.mppt_direction = 1;

ctrl.fault_flags = 0;

ctrl.soft_start_done = 0;

}

/*******************************************************************************

* Function: SyncBoost_VoltageLoop

* Description: Inner voltage regulation loop - call at 10-100kHz rate

*              (typically from timer interrupt)

*

* This implements a simple PI controller. For better performance,

* implement the Type III compensator from the control design document.

******************************************************************************/

void SyncBoost_VoltageLoop(void)

{

/* Get measurements */

SyncBoost_GetMeasurements(&ctrl.v_in, &ctrl.v_out, &ctrl.i_in, &ctrl.temp);

/* Check for faults first */

if (ctrl.v_out > VOUT_MAX) {

ctrl.fault_flags |= FAULT_OVP;

SyncBoost_PWM_Stop();

return;

}

if (ctrl.i_in > IIN_MAX) {

ctrl.fault_flags |= FAULT_OCP;

SyncBoost_PWM_Stop();

return;

}

if (ctrl.temp > TEMP_MAX) {

ctrl.fault_flags |= FAULT_OTP;

SyncBoost_PWM_Stop();

return;

}

/* PI Controller for output voltage regulation */

float error = VOUT_TARGET - ctrl.v_out;

/* Integral with anti-windup */

ctrl.integral_error += error * KI_VOLTAGE;

if (ctrl.integral_error > 0.5f) ctrl.integral_error = 0.5f;

if (ctrl.integral_error < -0.5f) ctrl.integral_error = -0.5f;

/* Calculate duty cycle adjustment */

float duty_new = ctrl.duty + KP_VOLTAGE * error + ctrl.integral_error;

/* Soft-start ramp */

if (!ctrl.soft_start_done) {

static float soft_start_duty = 0.0f;

soft_start_duty += 0.001f;  // Ramp up slowly

if (soft_start_duty >= duty_new) {

ctrl.soft_start_done = 1;

} else {

duty_new = soft_start_duty;

}

}

/* Thermal derating */

if (ctrl.temp > TEMP_WARN) {

float derate = 1.0f - (ctrl.temp - TEMP_WARN) / (TEMP_MAX - TEMP_WARN);

duty_new *= derate;

}

/* Update duty cycle */

ctrl.duty = duty_new;

SyncBoost_SetDuty(ctrl.duty);

}

# 9. Protection Implementation

## 9.1 Hardware Comparator for Fast OCP

For fastest overcurrent response, use the STM32 internal comparator:

/*******************************************************************************

* Function: SyncBoost_OCP_Init

* Description: Configure COMP1 for cycle-by-cycle overcurrent protection

*              COMP output connected to TIM1 BREAK input

******************************************************************************/

void SyncBoost_OCP_Init(void)

{

COMP_HandleTypeDef hcomp1 = {0};

__HAL_RCC_COMP_CLK_ENABLE();

hcomp1.Instance = COMP1;

hcomp1.Init.InvertingInput = COMP_INVERTINGINPUT_DAC1;  // Threshold from DAC

hcomp1.Init.NonInvertingInput = COMP_NONINVERTINGINPUT_IO1; // Current sense

hcomp1.Init.Output = COMP_OUTPUT_TIM1BKIN;  // Route to TIM1 break input

hcomp1.Init.OutputPol = COMP_OUTPUTPOL_NONINVERTED;

hcomp1.Init.Hysteresis = COMP_HYSTERESIS_MEDIUM;

hcomp1.Init.Mode = COMP_MODE_HIGHSPEED;

HAL_COMP_Init(&hcomp1);

HAL_COMP_Start(&hcomp1);

/* Set OCP threshold via DAC */

/* For I_limit = 15A with R_sense = 20mΩ: V_threshold = 0.3V */

/* DAC value = 0.3V / 3.3V * 4096 = 372 */

HAL_DAC_SetValue(&hdac, DAC_CHANNEL_1, DAC_ALIGN_12B_R, 372);

HAL_DAC_Start(&hdac, DAC_CHANNEL_1);

}

## 9.2 Fault Handling

/* Fault flag definitions */

#define FAULT_NONE      0x00

#define FAULT_OVP       0x01  // Output overvoltage

#define FAULT_OCP       0x02  // Overcurrent

#define FAULT_OTP       0x04  // Overtemperature

#define FAULT_UVLO      0x08  // Input undervoltage

/*******************************************************************************

* Function: SyncBoost_FaultHandler

* Description: Handle fault conditions - call from main loop

******************************************************************************/

void SyncBoost_FaultHandler(void)

{

if (ctrl.fault_flags == FAULT_NONE) return;

/* All faults: ensure PWM is stopped */

SyncBoost_PWM_Stop();

/* Log fault for debugging (implement your logging method) */

// UART_Printf("FAULT: 0x%02X\n", ctrl.fault_flags);

/* Attempt auto-restart after cooldown (optional) */

static uint32_t fault_time = 0;

if (fault_time == 0) {

fault_time = HAL_GetTick();

}

/* Wait 5 seconds before retry */

if (HAL_GetTick() - fault_time > 5000) {

/* Check if fault condition cleared */

SyncBoost_GetMeasurements(&ctrl.v_in, &ctrl.v_out, &ctrl.i_in, &ctrl.temp);

uint8_t can_restart = 1;

if (ctrl.v_out > VOUT_MAX * 0.9f) can_restart = 0;  // OVP not cleared

if (ctrl.temp > TEMP_WARN) can_restart = 0;          // Still too hot

if (ctrl.v_in < 20.0f) can_restart = 0;              // No input

if (can_restart) {

ctrl.fault_flags = FAULT_NONE;

fault_time = 0;

SyncBoost_ControlInit();  // Reset controller

SyncBoost_PWM_Start();

} else {

fault_time = HAL_GetTick();  // Reset timer, try again later

}

}

}

# 10. MPPT Algorithm Integration

## 10.1 Perturb & Observe Algorithm

P&O is simple but effective. It perturbs the operating point and observes power change:

- If power increased → continue in same direction

- If power decreased → reverse direction

/*******************************************************************************

* Function: SyncBoost_MPPT_PandO

* Description: Perturb and Observe MPPT algorithm

*              Call every MPPT_PERIOD_MS (e.g., 50ms)

*

* The algorithm adjusts the input voltage reference (v_ref_mppt).

* The voltage control loop then adjusts duty cycle to achieve this voltage.

*

* For a boost converter, increasing D DECREASES V_in (more power drawn).

******************************************************************************/

void SyncBoost_MPPT_PandO(void)

{

/* Calculate current input power */

ctrl.power_in = ctrl.v_in * ctrl.i_in;

/* Determine if power increased or decreased */

float delta_power = ctrl.power_in - ctrl.power_prev;

/* P&O logic */

if (delta_power > 0) {

/* Power increased - keep going same direction */

/* Do nothing, mppt_direction stays the same */

} else if (delta_power < 0) {

/* Power decreased - reverse direction */

ctrl.mppt_direction = -ctrl.mppt_direction;

}

/* If delta_power == 0, also keep same direction */

/* Apply perturbation to voltage reference */

ctrl.v_ref_mppt += ctrl.mppt_direction * MPPT_STEP;

/* Clamp to valid input voltage range */

if (ctrl.v_ref_mppt < 25.0f) ctrl.v_ref_mppt = 25.0f;

if (ctrl.v_ref_mppt > 60.0f) ctrl.v_ref_mppt = 60.0f;

/* Save for next iteration */

ctrl.power_prev = ctrl.power_in;

}

## 10.2 Incremental Conductance Algorithm

IncCond is more sophisticated, directly calculates MPP direction:

At MPP: dP/dV = 0 → dI/dV = -I/V

/*******************************************************************************

* Function: SyncBoost_MPPT_IncCond

* Description: Incremental Conductance MPPT algorithm

*              More accurate than P&O, especially with rapidly changing irradiance

******************************************************************************/

void SyncBoost_MPPT_IncCond(void)

{

static float v_prev = 0, i_prev = 0;

float dV = ctrl.v_in - v_prev;

float dI = ctrl.i_in - i_prev;

/* Avoid division by zero */

if (fabsf(dV) < 0.01f) {

/* Voltage hasn't changed much - use dI to decide */

if (dI > 0.01f) {

ctrl.v_ref_mppt -= MPPT_STEP;  // Increase power by decreasing V

} else if (dI < -0.01f) {

ctrl.v_ref_mppt += MPPT_STEP;

}

} else {

/* Calculate incremental conductance */

float inc_cond = dI / dV;

float inst_cond = -ctrl.i_in / ctrl.v_in;

if (fabsf(inc_cond - inst_cond) < 0.001f) {

/* At MPP - do nothing */

} else if (inc_cond > inst_cond) {

/* Left of MPP - increase voltage */

ctrl.v_ref_mppt += MPPT_STEP;

} else {

/* Right of MPP - decrease voltage */

ctrl.v_ref_mppt -= MPPT_STEP;

}

}

/* Clamp and save */

if (ctrl.v_ref_mppt < 25.0f) ctrl.v_ref_mppt = 25.0f;

if (ctrl.v_ref_mppt > 60.0f) ctrl.v_ref_mppt = 60.0f;

v_prev = ctrl.v_in;

i_prev = ctrl.i_in;

}

## 10.3 Main Loop Integration

/*******************************************************************************

* Function: main

* Description: Main application entry point

******************************************************************************/

int main(void)

{

HAL_Init();

SystemClock_Config();  // Configure for 100MHz or your target

/* Initialize peripherals */

SyncBoost_GPIO_Init();

SyncBoost_PWM_Init();

SyncBoost_ADC_Init();

SyncBoost_OCP_Init();

SyncBoost_ControlInit();

/* Start ADC with DMA */

HAL_ADC_Start_DMA(&hadc1, (uint32_t*)adc_raw, 4);

/* Start PWM (will start at 0% duty) */

SyncBoost_PWM_Start();

uint32_t last_mppt_time = 0;

uint32_t last_control_time = 0;

while (1) {

uint32_t now = HAL_GetTick();

/* Voltage control loop - run every 1ms (1kHz) */

/* For better performance, put this in a timer interrupt */

if (now - last_control_time >= 1) {

last_control_time = now;

SyncBoost_VoltageLoop();

}

/* MPPT loop - run every 50ms */

if (now - last_mppt_time >= MPPT_PERIOD_MS) {

last_mppt_time = now;

SyncBoost_MPPT_PandO();  // Or SyncBoost_MPPT_IncCond();

}

/* Fault handling */

SyncBoost_FaultHandler();

}

}

PART C: TESTING & VALIDATION

# 11. Bench Testing Procedures

## 11.1 Test Equipment Required

- DC Power Supply: 0-60V, 15A minimum (or solar panel simulator)

- Electronic Load: 0-150V, 10A minimum

- Oscilloscope: 4-channel, 100MHz+ bandwidth

- Current Probes: AC/DC, 20A rating

- Multimeters: For DC voltage/current verification

- Thermal Camera or Thermocouples: For temperature monitoring

## 11.2 Initial Power-Up Sequence

CRITICAL: Follow this sequence to avoid damage!

- Step 1: Verify all connections with power OFF

- Step 2: Check gate driver supply voltage (12V or 15V)

- Step 3: Apply low input voltage (25V) with NO LOAD

- Step 4: Verify PWM signals on oscilloscope before enabling output

- Step 5: Slowly increase input voltage while monitoring V_out

- Step 6: Add load gradually (start at 10% = 50W)

## 11.3 Dead-Time Verification Test

BEFORE applying high voltage, verify dead-time is working:

- Set up at low voltage (12V input, light load)

- Probe both gate signals (Q1 and Q2 gates)

- Zoom in on rising/falling edges

- Verify BOTH gates are LOW during transitions

- Measure dead-time duration (should be ~300ns)

If dead-time is missing or too short, DO NOT PROCEED - fix first!

## 11.4 Functional Tests

| Test | Procedure | Pass Criteria |
| --- | --- | --- |
| Startup | Apply 48V input, observe V_out | V_out ramps to 120V in < 100ms |
| Load Step Up | Step load 200W → 400W | V_out deviation < 5%, settles < 10ms |
| Load Step Down | Step load 400W → 200W | V_out deviation < 5%, no overshoot > 130V |
| Input Step | Step V_in 48V → 35V | V_out stays within ±2% |
| OCP Test | Short output briefly | Converter shuts down < 1µs |
| OVP Test | Disconnect load suddenly | V_out < 135V peak |

# 12. Oscilloscope Verification

## 12.1 Key Waveforms to Capture

### 12.1.1 Gate Drive Signals

Probe: Q1 gate (CH1), Q2 gate (CH2)

Verify:

- Complementary operation (when one is HIGH, other is LOW)

- Clean rising/falling edges (< 50ns)

- No ringing or overshoot > 20V

- Dead-time visible between transitions

### 12.1.2 Switch Node Voltage

Probe: Junction of Q1/Q2/L (switch node) to GND

Verify:

- Swings from ~0V to V_out (120V)

- Clean transitions (some ringing acceptable)

- Duty cycle matches expected D = 1 - V_in/V_out

### 12.1.3 Inductor Current

Probe: Current probe around inductor

Verify:

- Triangular waveform in CCM

- Peak-to-peak ripple matches calculation

- No saturation (current should not spike suddenly)

- Average value matches I_in = P_out / (V_in × η)

### 12.1.4 Output Voltage Ripple

Probe: V_out with AC coupling, 20MHz bandwidth limit

Verify:

- Ripple < 1% of V_out (< 1.2V peak-to-peak)

- Frequency matches switching frequency

- No large transient spikes

## 12.2 Thermal Verification

After 30 minutes at full load:

- MOSFET case temperature: < 80°C

- Inductor temperature: < 100°C

- Capacitor temperature: < 85°C

- PCB hot spots: < 100°C

# 13. Efficiency Measurement

## 13.1 Efficiency Test Setup

Measure input and output power simultaneously:

η = P_out / P_in = (V_out × I_out) / (V_in × I_in)

## 13.2 Efficiency Test Matrix

Measure efficiency at multiple operating points:

| V_in (V) | P_out (W) | Expected η | Measured η | Notes |
| --- | --- | --- | --- | --- |
| 25 | 100 | > 94% | ____ | High D |
| 25 | 500 | > 96% | ____ | Full load |
| 48 | 100 | > 95% | ____ | Light load |
| 48 | 500 | > 97% | ____ | Nominal |
| 60 | 100 | > 96% | ____ | Low D |
| 60 | 500 | > 98% | ____ | Best case |

## 13.3 Loss Breakdown Analysis

If efficiency is lower than expected, identify the loss sources:

- Conduction losses: Measure V_DS during ON state, calculate I²R

- Switching losses: Measure transition times, calculate E_sw × fs

- Gate drive losses: Measure gate charge, calculate Q_g × V_gs × fs

- Inductor losses: Measure DCR, calculate I²R + core losses

- Capacitor losses: Measure ESR, calculate I²_ripple × ESR

## 13.4 MPPT Efficiency

Verify MPPT is tracking the actual maximum power point:

- Use solar panel I-V curve tracer or simulator

- Set known MPP (e.g., V_mpp = 40V, I_mpp = 10A, P_mpp = 400W)

- Run MPPT algorithm

- Verify V_in settles within ±2% of V_mpp

- MPPT efficiency = P_extracted / P_mpp > 99%

--- End of Implementation Guide ---